Voltage regulator circuit

Cheng Jiang

South China University of technology (SCUT), College of Integrated circuit

202264680374

In this experiment, the ratio of output voltage to input voltage change, ripple rejection ratio, output resistance and other parameters are calculated by measuring the input and output voltage, input and output ripple voltage peak-to-peak, open-circuit voltage and load voltage of the voltage regulator circuit. This is used to determine the stability of the voltage regulator circuit.

ARTICLE INFO                    ABSTRACT

Submission Date:

November 6, 2023

Keywords:

Voltage regulator circuit

Ripple rejection ratio

1. Introduction

The so-called voltage stabilizing circuit refers to a circuit that can maintain a constant output voltage when the input voltage, load, ambient temperature, circuit parameters, etc. change. Voltage stabilizing circuit is a commonly used circuit in modern electronic technology. It is also an important component to ensure the normal and stable operation of electronic equipment. The voltage stabilizing circuit usually includes four parts: adjustment component, reference voltage circuit, sampling circuit and comparison amplifier circuit.

2. Objectives

1. Comprehend the composition and principle of voltage stabilizing circuits.

2. Implement the construction and debugging of voltage stabilizing circuits.

3. Understand and calculate relevant indicators of voltage stabilizing circuits.

3. Principle

3.1 Circuit composition

When the voltage of the power grid changes or the output load changes, the circuit that can keep the output voltage unchanged is called a voltage stabilizing circuit. DC regulated power supply is one of the most basic and commonly used instruments in electronic equipment. As an energy source, it can ensure the normal operation of electronic devices.

Figure 3.1-1 Block diagram of DC voltage stabilizing circuit

Figure 3.1-2 voltage stabilizing circuit with a fixed output of 5Vcircuit

Figure 3.1-3 Adjustable output voltage stabilizing

DC stabilized power supply generally consists of three parts: rectifier circuit, filter circuit and voltage stabilizing circuit, as shown in Figure 3.1-1. This experiment mainly discusses the DC voltage stabilizing circuit composed of linear integrated voltage stabilizing component 7805.

Figure 3.1-2 is a voltage stabilizing circuit with a fixed output of 5V composed of bridge rectifier, capacitor filtering and 7805 linear voltage stabilizing integrated block. Figure 3.1-3 is a bridge rectifier, capacitor filtering and 7805 linear stabilizing integrated block. It is an adjustable output voltage stabilizing circuit composed of voltage integrated blocks. The functions of each capacitor in the circuit are: C1 is the filter capacitor, and the empirical formula between this capacitance and the load current I0 is usually C1 = (1500~2000)·I0 (μF); C2 is to suppress the self-oscillation of the voltage regulator; C3 is the high-frequency noise bypass capacitor.

3.2 Main index parameters of voltage stabilizing circuit

Output voltage and its adjustable range: For adjustable regulated power supplies, the adjustable range of output voltage reflects the adaptability of the power supply's output application range.

Voltage stabilizing coefficient Sr: refers to the ratio of the relative change in output voltage UO to the relative change in input voltage UI of a DC voltage stabilizing power supply when the load current and ambient temperature remain constant, expressed as:

Ripple suppression ratio SSRPP: The voltage stabilizing circuit cannot completely eliminate the AC component, and the introduction of ripple suppression ratio describes the performance of the voltage stabilizing circuit. Defined as 20 times the logarithmic value of the ratio of input ripple voltage peak to peak and output ripple voltage peak to peak:

The larger the ripple suppression ratio, the stronger the ability of the voltage stabilizing circuit to eliminate AC components.

Output resistance RO: defined as the ratio of the change in output voltage to the change in output current when there is no change in input voltage and ambient temperature. In experimental measurements, the output resistance is usually regarded as an internal resistance similar to that of a power supply. By measuring the open-circuit output voltage UO and the voltage UO on the load RL, it can be substituted into the formula:

4.Equipment

AC power supply, digital multimeter, capacitor (0.1μF,1μF,470μF), resistance (100Ω,200 Ω,1KΩ), potentiometer(10KΩ), diode, three terminal voltage regulator (7805), dual trace oscilloscope, 9-hole plug-in board, input power supply, handheld multimeter, connecting line, etc.

| Figure 4.1-1 Input power supply | Figure 4.1-2 Handheld multimeter |
| --- | --- |
| Figure 4.1-3 9-Hole plug-in board and experimental circuit | Figure 4.1-4 Dual trace oscilloscope |

5.Procedures

5.1 DC voltage regulator circuit composed of 7805:

① Connect the circuit according to Figure 5.1-1, disconnect the circuit at point after connection, measure and record the waveform of UI (that is, the waveform of UA), and then switch on the circuit behind point A, observe the waveform of UO, if any oscillation should be eliminated. Adjust the RW, if the output voltage changes, the circuit works normally.

Figure 5.1-1 Adjustable output voltage stabilizing

② Measuring the output range of the stabilized power supply: Adjust the RW, monitor the output voltage UO waveform with oscilloscope, respectively measure the maximum and minimum output voltage of the regulator circuit, and the corresponding UI value. Measure the reference voltage of the regulator block (i.e. the voltage at both ends of the 100Ω resistor).

③ Observe ripple voltage: Adjust RW to UO = 5V, observe the waveform of the input voltage Ui of the voltage regulator circuit with an oscilloscope, and record the size of the ripple voltage, and then observe the ripple of the output voltage UO, and compare the two.

④ Measure the output resistance RO of the stabilized power supply: disconnect RL, and measure the voltage at both ends of RL with a multimeter, denoting it as UO’. Then access RL, measure the corresponding output voltage, recorded as UO, use the following formula to calculate RO:

6. Original Data

The recorded original data is shown in Table 6.1-1 and Table 6.1-2:

Table 6.1-1 Peak-to-peak measurement of input and output voltages and input and output ripple voltages

| Input voltage UI | Output voltage UO | Input ripple voltage peak to peak UOPP | Output ripple voltage peak to peak UIPP |
| --- | --- | --- | --- |
| 14.87V | 5.07V | 1.73V | 60mV |

Table 6.1-2 Measurement of open-circuit voltage and load voltage

|  | The first time | The second time |
| --- | --- | --- |
| Open-circuit voltage UO’ | 5.065V | 5.068V |
| Load voltage UO | 5.063V | 5.066V |

7. Analysis

7.1 Calculate the voltage stabilization coefficient

From the data in Table 6.1-1 and Table 6.1-2, we take relative variation of output voltage ∆UI and relative variation of input voltage ∆UO as half of UIPP and UOPP, respectively, and then we bring these data into Equation 1 (Output voltage UO = 5.07V, Relative variation of input voltage ∆UI= 0.865V, Input voltage UI= 14.87V, Relative variation of output voltage ∆UO = 30mv):

By calculation, we can know that Sr = 0.1017.

7.2 Calculate the ripple suppression coefficient

From the data in Table 6.1-1, we bring these data into Equation 2:

By calculation, we can know that SSRPP = 29.1979.

7.3 Calculate the output resistance (RL = 200Ω)

From the data in Table 6.1-2, we bring these data into Equation 3:

By calculation: we can know that RO1 = 0.079Ω, RO2 = 0.079Ω. By averaging the two measurements, we get RO = 0.079Ω.

8. Conclusion

For the voltage regulator circuit in this experiment, we measure the ratio of the relative change of the output voltage to the relative change of the input voltage Sr is 0.1017, the ripple rejection ratio SSRPP is 29.179, and the output resistance RO is 0.079Ω, indicating that the voltage regulator circuit is stable.

9. Appendix