Spectrometer Grating Diffraction

Cheng Jiang

South China University of Technology (SCUT), College of Integrated Circuit

202264680374

ARTICLE INFO                    ABSTRACT

This experiment uses a spectrometer and a grating for measurement. It uses a known grating to measure the wavelength of different lights and uses a known light wavelength to measure the grating constant of an unknown grating. Combined with the grating equation, the relative error between the final calculated result and the theoretical value is are less than or equal to 0.4%.

Submission Date:

October 23, 2023

Keywords:

Spectrometer

Grating equations

1. Introduction

Spectral measurement and spectral analysis are important means to study the composition of matter and study the structure of atoms and molecules. Grating is an important and indispensable dispersion element in spectrum measurement. It can divide the light emitted by the light source into spectral lines according to wavelength and arrange them in an orderly manner. A diffraction grating is composed of a large number of slits of equal width, equal spacing, and arranged in parallel. Generally, they can be divided into two categories: transmission gratings that work with transmitted light and reflection gratings that work with reflected light. Requirements for this experiment: Understand the principles of grating diffraction and study the characteristics of diffraction gratings; master the principles and methods of accurately measuring wavelengths with diffraction gratings; further familiarize yourself with the working principle of the spectrometer and the adjustment and use of the spectrometer.

2. Principle

2.1 Grating constants and grating equations

Figure 2.1-1 Diffraction grating

Diffraction grating is an optical element composed of a large number of slits of equal width, equal spacing, and parallel arrangement. Gratings used in the visible light region can have hundreds to thousands of slits per millimeter. Suppose the slit width is a, the width of the opaque part between adjacent slits is b, the slit spacing d = a + b is called the reciprocal of the grating constant, and 1/d is called the grating constant. As shown in Figure 2.1-1.

According to Fraunhofer's diffraction theory, when a parallel beam of wavelength λ is projected vertically onto the grating plane, the light wave will be diffracted at each slit, and the diffracted light from each slit will interfere at the superposition. The interference result is determined by the light Poor process. Because the spacing between the slits of the grating is equal, the optical path difference of the diffracted light beams along the θ direction from adjacent slits is dsinθ (As shown in Figure 2.1-1). θ is the angle between the diffracted beam and the normal line of the grating, which is called the diffraction angle.

Place a converging lens behind the grating so that the optical axis of the lens is parallel to the normal line of the grating (As shown in Figure 2.1-2). The lens will make the light with a diffraction angle θ on the plane shown in Figure 2.1-2 focus on point P on the focal plane, based on the principle of multi-beam interference, when θ satisfies the following formula, the main interference maximum will be generated, and point P is the bright spot:

where k is the series and d is the reciprocal of the grating constant. Equation (1) is called the grating equation and is the basic formula of diffraction grating.

Figure 2.1-2 Grating diffraction diagram

It can be seen from Equation (1) that θ = 0 corresponds to the central main maximum, and point P0 is the bright spot. The central main maximum is symmetrically arranged with ±1 level, ±2 level... main maximum. The actual grating has a large number of slits, and the smaller the slit width is, so when the light source that generates parallel light is a slender slit, the diffraction pattern of the grating will be thin and sharp bright lines arranged in parallel. These bright lines are actually the diffraction interference fringes of the light source slit.

2.2 Grating spectrum

When the incident light is polychromatic light, it can be known from the grating equation that for a grating with a given grating constant of 1/d, there are only central main poles of various wavelengths contained in the polychromatic light in the direction of k = 0, that is, θ = 0. They coincide with each other and form a bright central zero-order bright line on the focal plane of the lens. For other values of k, the main maxima of various wavelengths do not coincide, and thin sharp bright lines of different wavelengths appear in different directions of diffraction angles. The resulting spectrum is called a grating spectrum. Bright lines of various wavelengths with the same series number k are symmetrically arranged on both sides of the zero-order bright line in order from short wavelength to long wave to form a spectrum. k = 1 is the first-order spectrum, k = 2 is the second-order spectrum……, various wavelengths The thin sharp bright lines are called spectral lines. Figure 2.2-1 is a schematic diagram of the diffraction spectrum of a low-pressure mercury lamp. If the grating constant 1/d and the series k are known, the wavelength of the light wave can be determined by accurately measuring the diffraction angle of the spectral line. Conversely, the grating constant can also be determined from a known wavelength.

Figure 2.2-1 Schematic diagram of low pressure mercury lamp diffraction spectrum

2.3 Grating properties

Angular dispersion and resolution ability are two important characteristics of gratings. Diffraction grating can separate polychromatic light into spectral lines of different wavelengths on the focal plane of the lens according to wavelength, indicating that diffraction grating has dispersion effect. Due to the diffraction phenomenon, the spectral lines expand into wider bright stripes, thus limiting the resolving power of the grating. According to theoretical derivation, the dispersion capability of the grating can be characterized by angular dispersion D. As shown in Equation (2).

The above formula represents the angular distance between two single chromatographic lines of unit wavelength interval. By differentiating the grating Equation (2), the angular dispersion of the grating can be obtained.

It can be seen from Equation (3) that the smaller the grating constant, the greater the angular dispersion, and the higher the order of the spectrum, the greater the angular dispersion.

The resolving power R represents the ability of the grating to resolve spectral details. If the grating can just separate the two spectral lines λ and λ+dλ, then

According to Rayleigh's judgment, when the maximum light intensity of one spectral line coincides with the minimum of another spectral line, the two spectral lines can just be distinguished. From this we can derive Equation (5).

In Equation (5), N is the total number of slits of the grating, and k is the series of the spectrum. The number of N is very large, and it can be seen that the grating is an optical element with high resolution capability.

3.Equipment

Spectrometer, low pressure mercury lamp, grating plate, plane mirror, etc.

| Figure 3.1-1 Spectrometer | Figure 3.1-2 low pressure mercury lamp |
| --- | --- |

4.Procedures

4.1 Adjust the spectrometer

To study the grating characteristics and measure the spectrum using the grating equation dsinθ=kλ (k=0,± 1, ± 2,⋯) of the light vertically incident grating, it must satisfy:

1. Incident parallel light must be perpendicular to the grating surface.

2. The spectral lines on both sides of the zero order are of equal height. In order to meet the above two conditions, the spectrometer and plane grating need to be adjusted before measurement.

Adjustment before measurement:

1. Spectrometer adjustment requirements: the collimator produces parallel light, and the telescope is focused at infinity; the optical axis of the collimator and the optical axis of the telescope are perpendicular to the axis of rotation of the instrument, and the optical axis of the collimator and the optical axis of the telescope are on the same horizontal line.

2. Adjust the grating plane to be perpendicular to the optical axis of the collimator.

| Figure 4.1-1 | Figure 4.1-2 | Figure 4.1-3 |
| --- | --- | --- |

Specific method: Make the parallel light tube face the light source, adjust the parallel light tube to produce parallel light, and adjust the slit width to 1 mm~2mm. Rotate the telescope to align the crosshairs of the reticle with the center of the slit, see Figure 4.1-1. Fix the telescope and place the grating on the stage (As shown in Figure 4.1-2). According to visual inspection, try to make the grating plane bisect the line connecting "1" and "2" vertically, and "3" should be within the grating plane. Turn the vernier disk so that the grating is approximately perpendicular to the optical axis of the telescope. Use the self-collimation method to strictly adjust the leveling screws "1" and "2" under the stage (note that the tilting screws of the telescope cannot be adjusted) until the green cross emitted by the telescope and reflected by the grating plane is in the Figure 4.1-3 Position. At this time, the grating plane is perpendicular to the optical axis of the collimator and parallel to the instrument's axis of rotation, and then the vernier disk is fixed (note: it is not necessary to rotate the grating 180°).

3. Adjust the grating plane to be perpendicular to the optical axis of the collimator. Loosen the stop screw of the telescope, rotate the telescope, observe the positive and negative first-order spectral lines, and pay attention to whether the cross-hair intersection is in the center of each spectral line. If it is not in the center, adjust the screw "3" under the stage so that the centers of the positive and negative first-level spectral lines pass through the intersection of the crosshairs. At this time, the spectral lines on both sides are of the same height.

4.2 Measurement of spectral line wavelengths of low-pressure mercury lamps using plane diffraction gratings

After the spectrometer is adjusted as required, the stage and vernier disk must be fixed so that only the telescope can rotate around the main axis. First, turn the telescope left and right to fully observe the diffraction spectrum of the grating, and then start to measure the position of each spectral line. When measuring, you can start from the central bright stripe and measure to the left and right sides respectively. You can also measure from the left (or right) side to the right (left) side by rotating the telescope in one direction. After the measurement is completed, take the plane grating off the stage, but do not disturb the adjusted spectrometer.

Note: Before starting to measure the angle q, you must observe it for a week by moving the observation eyepiece. When it is determined that k=±2 level and the corresponding two yellow lights are clearly visible, the measurement can only be started. In addition, when k=±1, the angular size of the same spectrum deviating from level 0 should be approximately the same. It also has the same characteristics when k=±2, which can be used to check whether the grating is perpendicular to the incident light.

5. Original Data

5.1 Measure the wavelength of light from a known grating

As shown in Figure 5.1-1, the reciprocal of the grating constant of the known grating is 100 mm/line. Thus, the grating constant of the grating is 100 line/mm.

Figure 5.1-1 The known grating

It is known that the grating constant of the known grating is 0.01 line/mm, measure the green light of it. We record the data in Table 5.1-1.

Table 5.1-1 Determination of green light wavelength

| Frequency | First-order spectral line on the right |  | First-order spectral line on the left |  | Wavelength (nm) |
| --- | --- | --- | --- | --- | --- |
|  | Left window | Right window | Left window | Right window |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

It is known that the grating constant of the known grating is 0.01 line/mm, measure the first level yellow light of it. We record the data in Table 5.1-2.

Table 5.1-2 Determination of first level yellow light wavelength

| Frequency | First-order spectral line on the right |  | First-order spectral line on the left |  | Wavelength (nm) |
| --- | --- | --- | --- | --- | --- |
|  | Left window | Right window | Left window | Right window |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

It is known that the grating constant of the known grating is 0.01 line/mm, measure the second level yellow light of it. We record the data in Table 5.1-3.

Table 5.1-3 Determination of second level yellow light wavelength

| Frequency | Second-order spectral line on the right |  | Second-order spectral line on the left |  | Wavelength (nm) |
| --- | --- | --- | --- | --- | --- |
|  | Left window | Right window | Left window | Right window |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

5.2 Measure the unknown grating constant (the known green light wavelength is 546.07nm)

We know that the wavelength of green light is 546.07nm, and measure the grating constant of the unknown grating. We record the data in Table 5.2-1.

Table 5.2-1 Determination of grating constant of unknown grating by the known green light wavelength

| Frequency | First-order spectral line on the right |  | First-order spectral line on the left |  | Grating constant (line/mm) |
| --- | --- | --- | --- | --- | --- |
|  | Left window | Right window | Left window | Right window |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

6. Analysis

6.1 Analysis the wavelength of green light from a known grating

Next, let’s analyze in detail how the wavelength is calculated based on the diffraction angle.

From Table 5.1-1, we combine Equation (6):

Thus, we can get the Table 6.1-1.

Table 6.1-1 Diffraction angle of green light

| Frequency | 1 | 2 | 3 |
| --- | --- | --- | --- |
| diffraction angle |  |  |  |

Next, we put the data in Table 6.1-1 into Equation (1):

For Equation (1), k = 1, d =100 mm/line. Then we can calculate the wavelength of green light. So we get Table 6.1-2.

Table 6.1-2 The wavelength of green light

| Frequency | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Wavelength(nm) |  |  |  |

For the three measured values of green light wavelength, in order to reduce the measurement error, we find the average value of the green light wavelength to be 544.42 nm.

By comparing with the theoretical value of 546.07nm of green light wavelength, we use the calculation formula of relative error:

For equation (7),  is the absolute error and  is the true value.

Then, by equation (7), we can get:

6.2 Analysis the wavelength of first level yellow light from a known grating

From Table 5.1-2, we combine Equation (6):

Thus, we can get the Table 6.2-1.

Table 6.2-1 Diffraction angle of first level yellow light

| Frequency | 1 | 2 | 3 |
| --- | --- | --- | --- |
| diffraction angle |  |  |  |

Next, we put the data in Table 6.2-1 into Equation (1):

For Equation (1), k = 1, d =100 mm/line. Then we can calculate the wavelength of first level yellow light. So we get Table 6.2-2.

Table 6.2-2 The wavelength of first level yellow light

| Frequency | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Wavelength(nm) |  |  |  |

For the three measured values of first level yellow light wavelength, in order to reduce the measurement error, we find the average value of the first level yellow light wavelength to be 578.06 nm.

By comparing with the theoretical value of 579.07nm of first level yellow light wavelength, we use the calculation formula of relative error:

For equation (7),  is the absolute error and  is the true value.

Then, by equation (7), we can get:

6.3 Analysis the wavelength of second level yellow light from a known grating

From Table 5.1-3, we combine Equation (6):

Thus, we can get the Table 6.3-1.

Table 6.3-1 Diffraction angle of second level yellow light

| Frequency | 1 | 2 | 3 |
| --- | --- | --- | --- |
| diffraction angle |  |  |  |

Next, we put the data in Table 6.3-1 into Equation (1):

For Equation (1), k = 2, d =100 mm/line. Then we can calculate the wavelength of second level yellow light. So we get Table 6.3-1.

Table 6.3-1 The wavelength of second level yellow light

| Frequency | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Wavelength(nm) |  |  |  |

For the three measured values of second level yellow light wavelength, in order to reduce the measurement error, we find the average value of the second level yellow light wavelength to be 575.16 nm.

By comparing with the theoretical value of 576.96 nm of second level yellow light wavelength, we use the calculation formula of relative error:

For equation (7),  is the absolute error and  is the true value.

Then, by equation (7), we can get:

6.4 Analysis the unknown grating constant (the known green light wavelength is 546.07nm)

From Table 5.1-4, we combine Equation (6):

Thus, we can get the Table 6.4-1.

Table 6.4-1 Diffraction angle of green light

| Frequency | 1 | 2 | 3 |
| --- | --- | --- | --- |
| diffraction angle |  |  |  |

Next, we put the data in Table 6.4-1 into Equation (1):

For Equation (1), k = 1, λ =546.07 nm. Then we can calculate the grating constant of green light. So we get Table 6.4-1.

Table 6.4-1 The wavelength of green light

| Frequency | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Grating constant (line/mm) |  |  |  |

For the three measured values of green light grating constant, in order to reduce the measurement error, we find the average value of the green light grating constant to be 298.79 line/mm.

By comparing with the theoretical value of 300 line/mm of second level yellow light wavelength, we use the calculation formula of relative error:

For equation (7),  is the absolute error and  is the true value.

Then, by equation (7), we can get:

7. Conclusion

When we measure the green light passing through the known grating, we find that the wavelength of green light is 544.42 nm, and the relative error with the theoretical value of 546.07 nm is 0.30%; The wavelength of the first level of yellow light was measured as 578.06 nm, and the relative error with the theoretical value of 579.09 nm was 0.17%; The wavelength of secondary yellow light was measured as 575.16 nm, and the relative error with the theoretical value of 576.96 nm was 0.31%.

When we measure the grating constant of the unknown grating by the green wavelength, we measure that the grating constant of the unknown grating is 298.79 line/mm, and the relative error with the theoretical value of 300 line/mm is 0.40%.

Overall, our measurements are highly accurate and reliable.

8. Appendix